### GUIDANCE ON OPRED OFFSHORE CHEMICAL REGULATIONS (OCR) NON-COMPLIANCE NOTIFICATION FORM

Regulations:

Regulation 3(1) of The Offshore Chemicals Regulations 2002 as amended (OCR) states “no person shall use or discharge any offshore chemical … otherwise than in accordance with the terms or, and conditions attached to, a permit.”

Regulation 15(1A)(a) of the OCR states “The permit holder, or if there is no permit holder, the operator shall without delay provide the Secretary of State with information in such form and manner as the Secretary of State may direct, of an incident resulting in the use of, or the discharge of, an offshore chemical save in accordance with the terms and conditions attached to a permit”.  Notification to the Secretary of State is via the Offshore Petroleum Regulator for Environment and Decommissioning (OPRED).

A discharge is defined by the relevant regulations as an intentional emission of the chemical, or any of its degradation or transformation products, from an offshore installation.  A release is defined as any emission other than by way of a discharge.

- This OPRED OCR Non-Compliance Notification Form provides direction on how to report any discharge or use made without a permit being granted or any identified non-compliances against the terms and conditions included in a previously granted OCR Permit.

The form should be completed and sent to opred@energysecurity.gov.uk

- The only exception to this is any discharge that is done as a matter of urgency for the purpose of securing the safety of any person – under these circumstances relevant authorities must be notified using the OPRED Petroleum Operations Notice (PON1).

- The non-compliance notification form must not be used for reporting any release of chemical to sea. Chemical releases must be reported using the OPRED Petroleum Operations Notice (PON1) form.  Where a release has been made and reported via a PON1, any further release or un-permitted discharge of chemical from the same release point must continue to be reported by updating the original PON1.   These PON1 updates are required regardless of whether the release or un-permitted discharge is continuous or intermittent and must be continued until such time as a repair or remedy has been effected and the release or un-permitted discharge is permanently stopped.

Guidance on Permit Condition Notification Form Fields:

- Identity of Reporter: Contact details of person and company reporting must be provided.

- Permit/Installation Details: Full OPRED Permit number can be found on the OCR permit (if applicable); details of permit holder/operator, installation name and field name associated with the non-compliance must be included.

- Permit Condition Number (if applicable):  Each permit will contain conditions that must be complied with.  Conditions are numbered within the permit.  The specific permit condition number that is out of compliance should be included within this field.

- Date Non-Compliance Identified: This is the date that the non-compliance was first identified.  If this date differs from the date that the non-compliance event took place, then further details should be included within this field.

- Details of Non-Compliance: An explanation of the non-compliance should be provided within this section including any relevant dates, volumes, concentrations, chemical names, ranking etc.

- Cause of Non-Compliance: An explanation should be given as to why the non-compliance occurred.  If details are still being investigated and are not currently available, please do not delay sending notification – updated information can be forwarded at a later date.

- Steps taken to prevent re-occurrence: Details should be provided of the actions taken, or to be taken, to prevent a similar non-compliance during future operations.

- Is Non-Compliance likely to result in significant environmental impact: It is expected that following any non-compliance the permit holder should carry out a brief environmental impact assessment.  If this identifies that a significant environmental impact could occur, then full details should be provided.

- Is Non-Compliance Ongoing: If non-compliance is ongoing this should be identified along with an explanation.

  - For further information and/or guidance on any aspect of this form or notification of non-compliances please contact your assigned, OPRED Environmental Inspector or send an email to opred@energysecurity.gov.uk